6 His GroES7 prep

Buffer A: (amounts for 1 L)	Buffer B: (amounts for 1 L)

10 mM Na2HPO4  (1.42 g)	10 mM Na2HPO4 (1.42 g)

10 mM imidazole (0.68 g)	0.5 M imidazole (34.04 g)

0.5 M NaCl (29.22 g)	0.5 M NaCl (29.22 g)

pH 7.5	pH 7.5

Buffer C: (amounts for 1 L)	Buffer D: (amounts for 1 L)

50 mM Tris (6.05 g)	50 mM Tris (6.05 g)

pH 7.5	1 M NaCl (58.44 g)

pH 7.5

- Obtain cell paste from a 2 L growth, let sit on bench top to thaw

- Add Buffer A to 10X dilution, vortex to resuspend

  - Example: for 5 g pellet, add 50 mL Buffer A

- Pour resuspension into beaker, stir slowly overnight at 4°C

- Turn on centrifuge and spin at low speed for it to cool down

- Crack cells

- Pour lysate into centrifuge tubes, balance tubes, spin at 10,000 rpm for 1 hour

- During spin, equilibrate Ni-NTA column:

  - Pump Buffer A through peristaltic pump ~5 minutes, setting 3.5

  - Connect column and equilibrate with 10 column volumes of Buffer A (150 mL)

- Pour supernatant from spin into fresh beaker

- Take gel samples of pellet and supernatant

  - Supernatant gel sample: 30 µL sample + 10 µL of 4X

  - Pellet gel sample: Get scrape of pellet, resuspend in 150 µL of 50% glycerol, add 50 µL of 4X

- Load supernatant onto Ni-NTA column using pump, same setting, save flow through & take gel sample

- Wash column with 10 CV of Buffer A, save wash & take gel sample

- Transfer column to HPLC to elute protein:

  - Connect HPLC lines together with white connector, perform pump wash

  - Connect column using yellow connectors, run elution program (10 CV, from 0% Buffer B to 100% Buffer B)

- Run gel samples from steps above and HPLC fractions on SDS-PAGE

- Combine appropriate fractions and dialyze overnight into 2 L of Buffer C at 4°C with slow stirring

- Equilibrate FastQ column:

  - Pump Buffer C through peristaltic pump ~5 minutes, setting 3.5

  - Connect column and equilibrate with 10 column volumes of Buffer C (100 mL)

- Take protein out of dialysis, load onto FastQ column using pump, save flow through, take gel sample

- Wash FastQ column with 10 CV of Buffer C, save wash, take gel sample

- Transfer column to HPLC to elute protein:

  - Connect HPLC lines together with white connector, perform pump wash

  - Connect column using yellow connectors, run elution program (10 CV, from 0% Buffer D to 100% Buffer D)

- Run gel samples from steps above and HPLC fractions

- Combine appropriate fractions and dialyze overnight into 2 L of Buffer C at 4°C with slow stirring

- Equilibrate Superdex200 column overnight with 3 column volumes of Buffer C

- Take protein out of dialysis, take concentration with Bradford reagent:

  - Make two 2.0 mL tubes, each containing 1500 µL of Bradford reagent, let warm to room temperature before using

  - Add 50 µL of protein to one tube, start timer, turn on spec, let incubate 10 minutes

  - Take absorbance at 595 nm, use standard curve by spec to calculate concentration

  - Calculate target volume to concentrate protein to:

    - (concentration of protein)(current volume) = (20 mg/mL)(X-target volume)

- Concentrate protein to target volume using YM-10 concentrator (MWCO 10 kDa): 15 minute spins, 3000 rcf, 4°C, save flow through

- Once target volume is reached, re-take Bradford to confirm concentration using step 22, also take Bradford of flow through (should be very low absorbance)

- Pipette protein into 1 mL aliquots

- Spin 1 mL of protein in centrifuge in cold room, top speed, 30 minutes (spin each mL of protein just before loading onto column)

- Attach 1 mL loading loop to HPLC, wash in at least 6 mL of Buffer C

- Load protein into loop, run program

- Run gel of peak

- Once all protein has been run through Superdex200 column and collected, combine appropriate fractions

- Take concentration and determine target volume, concentrate protein, re-take Bradford using step 22, final concentration should be ~10 mg/mL

- Aliquot protein for a final volume of 300 µL per tube, add 100% glycerol to a final concentration of 20% (do not use more than 20% glycerol final concentration!)

- Label tubes, freeze in -80°C freezer

When freezing protein for MDH/ATPase experiments, final concentration (after adding glycerol) must be at least 7 mg/mL