GroES single prep

GroES single works as a heptamer for dialysis (so can use normal dialysis tubing), but works as a monomer for concentration (must use small MWCO concentrators)

Buffer A: (amounts for 1 L) 	Buffer B: (amounts for 1 L)

50 mM Tris-HCl (7.88 g)	50 mM Tris-HCl (7.88 g)

0.1 mM EDTA (0.42 g)	0.1 mM EDTA (0.42 g)

1 mM DTT (1 mL of 1 M)	1 mM DTT (1 mL of 1 M)

pH 7.4	1 M KCl (74.55 g)

pH 7.4

- Obtain cell paste from a 2 L growth, let sit on bench top to thaw

- Add Buffer A to 10X dilution, vortex to resuspend

  - Example: for 5 g pellet, add 50 mL Buffer A

- Pour resuspension into beaker, stir overnight at 4°C

- Turn on centrifuge and spin at low speed for it to cool down

- Crack cells (separate protocol)

- Pour lysate into centrifuge tubes, balance tubes, spin at 10,000 rpm for 1 hour

- During spin, equilibrate DEAE column:

  - Pump Buffer A through peristaltic pump ~5 minutes, setting 3.5

  - Connect column and equilibrate with 10 column volumes of Buffer A (100 mL)

- Pour supernatant from spin into fresh beaker

- Take gel samples of pellet and supernatant

  - Supernatant gel sample: 30 µL sample + 10 µL of 4X

  - Pellet gel sample: Get scrape of pellet, resuspend in 150 µL of 50% glycerol, add 50 µL of 4X

- Load supernatant onto DEAE column using pump, same setting, save flow through & take gel sample

- Wash column with 10 CV of Buffer A, save wash & take gel sample

- Transfer column to HPLC to elute protein:

  - Connect HPLC lines together with yellow connector, perform pump wash

  - Connect column using white connectors, run elution program (20 CV- 200 mL, from 0% Buffer B to 100% Buffer B)

- Run gel samples from steps above and HPLC fractions on gel to determine protein purity

- Combine appropriate fractions and dialyze overnight into 2 L of Buffer C at 4°C with slow stirring

Buffer C: (amounts for 1 L) 	Buffer D: (amounts for 1 L)

50 mM K acetate (4.91 g)	50 mM K acetate (4.91 g)

0.5 mM EDTA (0.21 g)	0.5 mM EDTA (0.21 g)

pH 4.5	1 M KCl (74.55 g)

pH 4.5

Buffer E: (amounts for 1 L)

50 mM Tris-HCl (7.88 g)

200 mM NaCl (11.69 g)

pH 7.4

- Take protein out of dialysis (there will be a lot of precipitation)

- Put a drop on pH paper to make sure the pH is around 4.5

- To remove the precipitated contaminant proteins, spin protein in 50 mL tube(s) at 4000 rpm, 10 minutes, pour protein in new tube

- Equilibrate FastS column:

  - Pump Buffer C through peristaltic pump ~5 minutes, setting 3.5

  - Connect column and equilibrate with 10 column volumes of Buffer C (100 mL)

- Load protein onto FastS column using pump, save flow through & take gel sample

- Wash FastS column with 100 mL of Buffer C, save wash & take gel sample

- Transfer column to HPLC to elute protein:

  - Connect HPLC lines together with yellow connector, perform pump wash

  - Connect column using white connectors, run elution program:

    - 10 CV (100 mL) gradient from 0% D to 50% D

    - Step to 100% D, run 20 mL

- Run gel samples from steps above and HPLC fractions on gel to determine protein purity

- Combine appropriate fractions and dialyze overnight into 2 L of Buffer E at 4°C with slow stirring

- Take protein out of dialysis, take concentration with Bradford reagent:

  - Make two 2.0 mL tubes, each containing 1500 µL of Bradford reagent, let warm to room temperature before using

  - Add 50 µL of protein to one tube, start timer, turn on spec, let incubate 10 minutes

  - Take absorbance at 595 nm, use standard curve by spec to calculate concentration

  - Calculate target volume to concentrate protein to:

    - (concentration of protein)(current volume) = (10 mg/mL)(X-target volume)

- Concentrate protein to target volume using YM-3 concentrator (MWCO 3 kDa): 30 minute spins, 3000 rcf, 4°C, save flow through

- Re-take Bradford to confirm concentration, also take Bradford of flow through (should be very low absorbance)

- Add 100% glycerol to a final concentration of 20% (do not use more than 20% glycerol final concentration!)

- Label tubes, freeze in -80°C freezer